LynxSpring, Inc.

Onyxx CAN File System

OCANFS Design

Bryan Wilcutt

8/16/2016

The architectural and implementation design of the Onyxx Closed Area Network Virtual File System.

Contents

Introduction	2

Purpose	2

Audience	2

Overview	2

Design Considerations	3

Assumptions	3

System Environment	4

Design Methodology	4

Architecture	4

Data Flow Diagram	4

OCANFS	5

FUSE	5

NFSv4	5

CAN Driver	6

OCAN Protocol	6

EFF	6

ATTN	6

OCAN Messages	7

Tree Hierarchy Overview	9

Testing Environment	9

Build Environment	9

# Introduction

This document covers the design and implementation of the Onyxx CAN Virtual File System (OCANFS).  The OCANFS extends a CAN bus based IoT Onyxx device to a virtual file system that can be easily accessed from any platform supporting simple file I/O and NFSv4.    Lynxspring has created Lynxspring Onyxx platform as an IoT solution that addresses the management of intelligent devices and non-intelligent devices.

## Purpose

The Onyxx CAN File System extends analog, digital and user input devices from the Onyxx CAN platform to a local Linux based file system.  This file system can be access locally at the Onyxx host device or remotely via NFS.  By deploying a file system to allow CAN bus point access the user and software are removed from the necessary, complicated management of high speed data.   Accessing any CAN bus data point or setting is now as simple as reading and writing a text file from the perspective of most software.

Included with this implementation is an auto-discovery process by which the OCANFS software manages guest devices that extend beyond the host.

## Audience

The audience for this documentation is internal, technical individuals wanting to get an understanding of the OCANFS or for those tasked with maintaining this software.

## Overview

The following diagram outlines how the OCANFS operates within the Linux network and file system frameworks.   For networking applications, OCANFS uses the FUSE (Filesystem for USErs) subsystem which is ubiquitous to most Linux installations.  The networking component is optional, of course, as OCANFS can be access on the local host Onyxx device through terminal.

# Design Considerations

Several approaches were taken in considering the overall design of this application.  Developing the application using sysfs and a Platform Driver was abandoned due to the need to extend the sysfs file system using NFS which could not be successfully performed.  The second attempt was a standard user space device driver [UIO] however issues surfaced during the investigation phase as to the access the UIO driver would have to the CAN bus driver system.

Based on these findings the software was developed in user spacing using the FUSE file system.  The approach avoided any pollution of OS drivers, was easier to implement and will be easier to maintain in the future.

## Assumptions

The premise assumption for this project was to extend CAN bus data points to a sharable file system that could be mounted remotely.  Remote sharing of the file system would allow numerous Onyxx devices to be coalesced into a single location for management.

Another important assumption is the complete customization of CAN messages between the CAN micro-processors and the OCANFS system.  The OCANFS uses a customized CAN message format that allows management of multiple Onyxx devices (a host and multiple slaves) and numerous digital, analog and UI data points.  The protocol is described herein.

## System Environment

The OCANFS driver runs under Linux in User Space.  It is started by the systemctl system and includes necessary dot.service files to allow this to occur.  See the systemctl manpages for further information on this system.

The OCANFS uses minimal amount of static memory and does not directly use dynamic memory allocation; all resources are statically defined to avoid memory issues and to speed up operation.  The total amount of memory required by the software is approximately 3K of dynamic RAM per managed device.  The system can run in background mode but must be no more than 2-degrees less than user space software priority.

The software may also be built wth debug information printed on a terminal.  This is done so by defining the TEST_MODE flag in the ocanfs.h file.

## Design Methodology

The OCANFS system was designed as a simple M. Jackson Data application.   Software design followed standard Sarson-Gane methodology as seen below.

# Architecture

## Data Flow Diagram

The OCANFS system creates two threads, one for writing to the CAN bus of any active UI point, another to read from the CAN bus and to update the FUSE File System.   Data is read and written to the internal storage ram space for all data points.  Only UI points are read-write enabled, all other points are read-only.

### OCANFS

The OCANFS  component starts the FUSE, Reader and Writer threads using POSIX threads (pthread).  Each thread runs indefinitely or until a control variable is set that indicates each one should end.

The reader task is focused on reading CAN data, parsing the CAN ID field using the Onyxx Protocol, storing data, and serving the data to the FUSE virtual file system.  In contrast, the writer task is focused on updating any UI point back to the CAN bus.   Undiscovered UI points are not updated.  Once a UI point is updated the writer task continuously broadcasts the UI change to the CAN bus per the CAN standard.  The cadence of this broadcast is every ~500ms.

### FUSE

FUSE is provided by the operating system kernel and interfaces with the OCANFS system through the FUSE API.  Since the FUSE system is unaware of our specific storage data, the OCANFS API provides specific functions to read, write, open, close, list directories, etc.

### NFSv4

FUSE interfaces only with NFSv4, it does not work with earlier version of NFS.  When mounting an OCANFS system remotely the mount command must specify “-t nfs4”.  Other setup issues exist when using NFSv4 that differ from earlier versions.  NFSv4 setup and configuration is beyond the scope of this document.

## CAN Driver

The CAN Bus Driver is provided by the operating system.  The driver, known as c_can, provides the interface between the CAN bus and the described protocol.

Manually configuring the CAN bus is done by:

root@onyxx:/etc# sudo modprobe can

root@onyxx:/etc# sudo modprobe can_raw

root@onyxx:/etc# sudo modprobe can

root@onyxx:/etc# sudo ip link add dev can0 type can

root@onyxx:/etc# sudo ip link set up can0

Alternatively, using the ‘vcan0’ device allows the software to be tested in a non-CAN environment such as a Linux PC.

## OCAN Protocol

The CAN frame returned by the CAN driver contains a 32-bit (EFF) value that is defined by the bit-fields described below,

### EFF

The EFF field sets the Extension Frame Format which indicates this frame has a 32-bit ID field as described. Non-EFF CAN messages are ignored by the OCANFS.  It is not necessary to set this field within the software since the CAN driver manages the field directly.

### ATTN

The ATTN attention field indicates to the OCANFS system that the contained frame has an active CMD field.   When this field is set high, the CMD field is read for a specific command as described in the CMD section.  When the field is set low, the frame is treated as a normal to-and-from object.

Device ID

This field indicates which device this frame either comes  from or is directed towards.  Note that Device ID #0 always indicates the host Onyxx device.   Device IDs are assigned to guest devices through the IAM/UAR messaging sequence, see below.   No guest device should assume they have a device ID without first negotiating for that device ID.

D0 Point & A0 Point

Indicates which D0 or A0 is focused.  D0 and A0 points begin at 1 (one) and not 0 (zero).  Zero is reserved and indicates no point is present in the frame.  When D0 is indicated, the payload contains a single byte field containing either 0 (zero) or 1 (one).   Since Digital points are “on or off”, no other value is indicated.  For A0 points the payload portion of the CAN message will contain a 2-byte integer field.  Both fields are read and stored in the file system under:

/mnt/ocanfs/onyxx{n}/d0/d.{p}/value

/mnt/ocanfs/onyxx{n}/a0/a.{p}/value

Where n = 0 (host) to the number of recognized guests and p indicates the point number (1-8).  The value file contains the binary representation of data from the CAN bus indicated for this point.

UI Point

Universal Input (UI) points or readable and writable using a 2-byte integer, 16-bit payload.  Similarly to A0 and D0, UI points are also stored in the file system under:

/mnt/ocanfs/onyxx{n}/ui/u.{p}/value 	Current value

/mnt/ocanfs/onyxx{n}/ui/u.{p}/mode	Current mode

The mode file is not communicated to the CAN bus and is only present as an informational helper.  Only the values of resistance, current or voltage may be stored in this file.

### OCAN Messages

The OCANFS defines several specific messages to allow out-of-band communication between the Host and any connected Guest device.  These messages allow for connection and auto-discovery.

#### MSG_IAM (0x00)

Sent Guest->Host upon startup of a guest device.  The guest shall continue to send this request every 200ms until a reply is returned from the Host (UAR).  This message broadcasts the guest’s unique MAC address which allows the host to return an new or existing device ID.  An existing device ID is returned only if the guest had previously communicated to the host.  This feature allows guests to go off the CAN bus for a period of time (power outage, maintenance, etc.) but return with little disruption.

##### Ladder Diagram

#### MSG_UAR (0x01)

Sent Host->Guest, the UAR message is returned in response to an IAM message received from a guest.  The UAR message contains the original MAC address passed via the IAM message along with an assigned device ID.  The returned device ID must be used by the guest for all further communications.

See MSG_IAM for usage.

#### MSG_WAY (0x02)

Sent Host->Guest, the WAY (Who Are You) message requests a guest to identify themselves.  The identifier of this message is the previously given MAC address.  The host may request a device to return their device ID based on receiving this message as a command.  The returned message is an IAM.

##### Ladder Diagram

#### MSG_RESET (0x03)

Sent Host->Guest, this message instructs a guest, identified by their device ID, to perform an internal reset.

##### Ladder Diagram

### Tree Hierarchy Overview

The follow section demonstrates a sample OCANFS directory tree as built with a host device and a single guest device.

## Testing Environment

Testing the OCANFS system can be accomplished by instantiating the OCANFS using the vcan0 device.

# ocanfs vcan0 &

Messages may be sent or received from the OCANFS by using the can-utils, available as a Linux add-on.

# sudo apt-get install can-utils

The functions of interest are cansend and candump.  By using the cansend utility, messages can be sent to OCANFS to study how it behaves.

# sudo cansend 40000000#00

This example sends an ID field with ATTN set high and CMD=0x00 (IAM).  The returned value is a UAR message with an assigned device address.

## Build Environment

OCANFS was built using Eclipse MARS and gcc.  Both a Debug and Release model has been defined and the software may be available through BitBucket (Onyxx Device Drivers).  Building the software can be accomplished by importing the onyxx_can project into Eclipse and simply building.  Alternatively, it is possible to go to either the Debug or Release directory of onyxx_can and,

# make clean

# make all